THE TRUTH ABOUT

HARD DISK HEAD PARKING PROGRAMS

by Dan Goodell

Second Edition, April 1994

If you've been working with PC's for awhile, you've probably

heard the advice to "park" your hard drive before turning off

your computer.  This is the process of moving your hard drive's

heads to a designated cylinder (track).  In use, the drive heads

actually float above the disk surface on a cushion of air.  When

the disk stops spinning, the airflow ceases and the head "lands"

on the disk surface.  Winn Rosch, in his book "The Hardware

Bible", describes this as more of a "controlled crash", with the

potential for damage to any data stored at the tracks the heads

land on.  Thus, it's recommended that the heads be moved to a

cylinder where data is not likely to be stored, designated the

"landing zone".  Most modern drives are self-parking, moving

their heads to the landing zone automatically when power is

turned off.  Older drives can use a parking program to move the

heads just before the computer is turned off.

Many self-parking drives park only when power is turned off.

However, there is a body of opinion that recommends also parking

the heads anytime the system will be idle for awhile, even if

it's not turned off.  Then, in case of a bump or physical shock

to the hard drive, there is less danger of the heads bouncing

against the disk surface and damaging data.  The risk of this

happening is probably quite small, but if you are inclined to

take such precautions, parking programs may find a use even on

self-parking drives.  Note, however, that some drives now

include a timed idle-parking feature in the drive electronics,

which would make any third-party parking utility redundant.

Head parking programs simply move your drive's heads to a safe

track.  They do not control the drive motor, which may continue

to spin.  Some drive models may include a feature which turns off

the drive motor when the heads are moved to a certain cylinder -

one of this review's test systems included such a drive.  Though

not widely recognized or used, some IDE drives also feature a

"sleep mode" - by directly manipulating the Input/Output port

registers, the drive motor may be turned off while power is still

applied to the rest of the system.  Note that control of the

drive motor is a feature of the drive, not any parking program.

I am aware of two programs that manipulate the "sleep mode" of

such IDE drives, but they are not reviewed here - this review

considers only head parking programs, which should work on any

hard drive.

All of these parking programs use standard bios routines.  I

found no indication that these routines could directly cause

damage to the drive, although using them to park at the wrong

track would defeat the purpose of the program.

Some manufacturers may advise against using parking utilities

with their drives.  If in doubt, consult the manufacturer of your

drive and consider his advice.

If you use or have searched for a public domain or shareware hard

drive parking program, you may be aware that there are numerous

choices available.  Yet, evaluating these for suitability for

your purpose is not easy.  Furthermore, the screen responses of

many of them are so spartan that they can leave the user wondering

exactly what has been done when the program finishes.  The screen

displays are not always correct, and digging into the program

code reveals some surprises - there are definite differences

among these programs!

Most parking programs use a system bios routine that reports the

last usable cylinder on the drive that is available to DOS.  But

from here, programs differ.  Some assume this is the absolute

last track on the disk and park the heads there.  Others assume

there is one further cylinder for parking or diagnostic purposes,

normally hidden from the system for data storage.  Still others

assume there are even more hidden cylinders.  None of these

approaches is more correct than the others because examples of

any may exist in one system or another.

Most programs do not test for errors in these assumptions.

Before condemning the programs, though, realize that their job

is made more difficult by systems and drive controllers that are

remarkably inconsistent.  Ask the system bios for the parameters

of a nonexistent hard drive and one system will correctly report

the drive doesn't exist, while another returns spurious results

without warning of an error.  Try to move the heads to a non-

existent track and one system flags an error and does nothing,

but another system simply moves to the last valid track and

doesn't call it an error.  As a result, programs can encounter

a wide variety of responses among different systems.  A program

may not move the heads at all, may park at the wrong cylinder,

or may simply crash the system.  While the latter would probably

be evident, the user may not notice the other problems.

Remember, many of these programs will claim successful parking

even if an error occurs.  It is important for the user to choose

wisely, selecting a program that makes the right assumptions for

his particular system.

So how do you go about choosing the right parking program?  If

your drive's manufacturer provides a parking program, you can

probably presume he has verified that it works properly with

his drive.  If not, you will be looking at third-party programs.

I rounded up a number of public domain, shareware, and other

programs, testing each of them on a variety of PC's, and also

disassembling the program code in several of them.  Testing

included trapping hard disk interrupt calls to see exactly what

commands were being sent to the drive controller.  Before

selecting a parking program, explore how your system responds

to the bios routines the program will be using.  Then choose

among the programs that your system will respond to properly.

Use the accompanying utility, SEEK80.COM, to explore your system.

This program will check the first hard drive in your system and

display the number the bios routine will return to a parking

program as the "last" cylinder.  In a system with multiple hard

drives, SEEK81.COM and SEEK82.COM will do the same for the second

and third drives, respectively.  Parking utilities operate on

physical disk drives, not on logical drives.  In other words,

the first hard drive is referred to as drive 80 (hexadecimal),

not drive C:.  A track number may be input as an optional command

line parameter to the SEEK programs.  In that case the utility

will also attempt to move the heads on the appropriate hard drive

to the requested cylinder, reporting any errors.  This may help

identify whether your drive has hidden tracks beyond the last

cylinder DOS thinks is available.  Note the bios routine limits

the track number to the range from 0 to 1023.  By studying the

responses provided by the SEEK programs you should be able to

predict how the various parking programs reviewed here will

respond to your system.

Head parking programs fall into two basic categories: immediate-

action utilities and memory-resident time-delay parkers (TSR's).

Immediate-action programs move the heads immediately to an inner

track in preparation for powering down or an idle period when

the user is away from the computer.  Alternatively, a TSR will

monitor hard drive activity, and after a period of inactivity

the heads will be moved to an inner track.  Note, however, that

you could turn off your computer before a TSR's timeout period

expires, so if your drive is not a self-parking type, an

immediate-action utility would still be useful before turning

off your computer.

Table I lists the programs in this review, which includes one

that I wrote.  The first eight are TSR's, while the rest are

immediate-action types.  There may well be additional programs

around with the same or similar names.  Since so many programs

with similar names are available, the table includes the authors

to further identify the programs.  Registration costs are shown

for shareware programs.  Public domain and freeware programs are

shown with $0 cost.  Some programs do not show a cost - these do

not appear to be distributed as public domain or shareware.

HDPARK-1 and PARK-3 were evidently bundled with some systems,

SHIPDISK was on an IBM Diagnostic Disk, PARK-7 is distributed

with the retail "PC Tools" package, and PARK-6 is distributed

with "Spinrite" and has been attributed to Steve Gibson as the

author.

TABLE I - Programs Reviewed

Filename       Size  Filedate   Ver.  Cost  Author

------------   ----  --------   ----  ----  -------------------

TIMEPARK COM    477   3-19-86   1.1    $0   Alpha Computer Serv

AUTOPARK COM    943   5-03-86          $0   Carl Hayes

VIDPARK  COM   1296   5-27-87   1.05   $0   Steven Georgiades

PDTIMPRK COM    569   9-02-87          $0   Dick Flanagan

TPB      COM   3368   6-06-88   2.0   $15   George Somers

TESRPARK COM** 6703   7-25-88   1.0    $0   Tesseract Dev Team

PARKTIME COM** 2417  10-31-90   4.C    $0   D. Idaho Long

DMGPARK  COM   1627   2-19-94   2.32   $0   Dan Goodell

------------   ----  --------   ----  ----  -------------------

SHIPDISK COM    436   6-03-83               IBM

DISKPARK EXE   1408   2-27-85   1.00   $0   Simcha Lerner

PARKAL   COM    571  10-15-85   1.0    $0   Skip Gilbrech

PARK-1   COM*   224  12-07-85          $0   Alpha Computer Serv

HDPARK-1 COM*   684   1-30-86   1.0         Telex

PARK-2   COM    663   5-03-86          $0   Carl Hayes

PARK-3   EXE   1157   5-21-86               Sumo Systems

PCPARK10 COM   2184   6-12-86   1.0    $0   Raintree Computer

PARK-4   COM    372  10-12-86   1.2    $0   SoftPatch

PARK-5   COM     49   2-06-87          $0   (anonymous)

PARKIT-1 COM* 13824   9-25-87   1.1b   $0   Andrew Appel

PKANYD   EXE   7403  12-09-87          $5   David Scheall

NJPARK   EXE   1255   6-11-88   1.00   $5   Mike Blaszczak

HDPARK-2 COM   4372   6-07-89   2.00   $1   Daniel Stasinski

PARK-6   COM    505   6-01-90               attr. Steve Gibson

PARKIT-2 EXE  19840   1-31-91   1.1    $7   EC Systems

PARK-7   COM    501   9-21-91               Central Point Softw

PARKHEAD EXE  12466   1-03-93   1.1   $19   Gary Tessler

*  There are several programs named PARK, HDPARK, and PARKIT.

To differentiate among them, they are hereafter referenced

with numeric suffixes in chronological order.

** PARKTIME and TESRPARK support both TSR and immediate-action

use.

Table II summarizes the operating features of these programs.

For each program I identified how many hard disks it will park,

what parking track it uses, and whether it checks for track seek

errors.  Immediate-action types are annotated as to whether they

lock the system when finished.  TSR's are annotated with the size

of the resident code and the timeout period range.  There are

also a couple potential problem areas, discussed in further

detail below.

TABLE II - Operating Characteristics

Program   No of  XT   AT   Locks  TSR    TSR    Error Math Sys

disks Park Park  System Size   Time   Check Prob Prob

--------  ----- ---- ----  ------ ----  ------  ----- ---- ----

TIMEPARK    2    +1   +1           640  1m-9m    No   No   Yes

AUTOPARK   1-2  find find          592  30s      Yes  No   Yes

VIDPARK    1-2   +1   +1           784  1m-1h*   No   No   Yes

PDTIMPRK    2    +1   +1           400  1m-9m    No   No   Yes

TPB         2    +0   +0          1280  1m-9m*   No   No   Yes

TESRPARK   all   +1   +1     No   5456  1m-9m*   No   No   No

PARKTIME    1    +1   +1     No*   800  10s-9m*  No   No   Yes

DMGPARK    1-2   +1*  +1*          592  1s-1h*   Yes  No   No

--------  ----- ---- ----  ------ ----  ------  ----- ---- ----

SHIPDISK   1-2  +50  +50     No                  No   No   Yes

DISKPARK   all   +0   +0    Temp                 Yes  No   No

PARKAL     1-2   +0   +0    Temp                 Yes  No   No

PARK-1      1    +1   +1    Perm                 No   Yes  No

HDPARK-1   1-2   +1   +1     No                  Yes  No   No

PARK-2     1-2  find find    No                  Yes  No   Yes

PARK-3     1-2   +0   +0     No                  Yes  No   No

PCPARK10   all   +1  CMOS   Perm*                Yes  No   Yes

PARK-4     1-2   +1   +1    Temp                 No   No   No

PARK-5      2    +1   +1     No                  No   No   No

PARKIT-1    ?    +1   +1    Perm                  ?   No   No

PKANYD      2    +0   +0     No                  No   No   No

NJPARK     all   +8   +8    Temp                 No   No   No

HDPARK-2   1-4   +1   +1    Temp                 No   No   Yes

PARK-6     all   +1   +1    Temp                 No   No   No

PARKIT-2    1    +2   +2    Temp                 Yes? Yes  No?

PARK-7     1-2   +0   +0     No                  No   No   No

PARKHEAD   1-2  find find    No                  Yes  No   Yes

* (Note: Installation characteristics; see text for options.)

DISCUSSION OF OPERATING CHARACTERISTICS

---------------------------------------

NUMBER OF DISKS:

Table II uses the following codes:

1   Program parks only one disk.

1-2  Program checks number of disks and parks up to two, if

present.  Note: HDPARK-2 will park up to 4 disks.

2   Program assumes two disks without checking, tries to park

both.

all  Program checks number of disks and parks all of them.

Except for HDPARK-2, the programs that check for the number of

drives get that number from a call to bios interrupt service

13h, function 08h, (INT_13/08h).  HDPARK-2 instead sequentially

asks for the parameters of consecutive disks and assumes that a

disk exists if no error is returned.  (Not always true - see

Appendix I.)

Many of these programs attempt to park a nonexistent second

disk.  This will not harm a system that has only one disk

installed.  The extra parking call is simply ignored by the

system.

PARKING TRACK:

Most programs call INT_13/08h ("Get Parameters") to determine

the number of tracks available on the disk.  Some assume this

is all and use this number as the parking track (+0).  Others

assume the parking track is one cylinder further inside and

increment this number by one (+1) to use as the parking track.

PARKIT-2 (+2) and NJPARK (+8) bump the number even further.

Since the bios routine accepts only track numbers up to 1023,

on disks approaching 1024 cylinders some of these programs may

wrap-around and park the heads at a beginning cylinder.

AUTOPARK and PARK-2 use a different technique, relying on seek

errors to find the actual innermost cylinder, though this does

not work in systems that don't return seek errors.

PARKHEAD relies on the bios service INT_13/04h to find the

innermost cylinder.  This is a test to verify a data sector,

so this method may not allow the program to use a true parking

track which has not been formatted as a data track.  On the 386,

with 1024 cylinders, there were no unformatted tracks, which

sent PARKHEAD into a continuous loop, wrapping back around to

cylinder 0 and never finding an unformatted track.

Only PCPARK10 uses the CMOS Landing Zone data in 286, 386, and

486 systems.  However, many modern drives ignore the CMOS LZ,

so not all users bother to even set it.  If you use PCPARK10,

make sure the CMOS LZ is set properly.

DMGPARK allows the track number to be overridden on the command

line, so the landing zone can be manually entered, if necessary.

Many TSR's find the parking track anew each time they are called

upon to park.  On the other hand, AUTOPARK, PDTIMPRK, and DMGPARK

find the track number when installed and then save it for future

reference.  In most cases, this difference is inconsequential.

LOCKING OF SYSTEM:

If you are going to be away from the system for awhile without

turning the power off, programs that permanently lock the system

will require rebooting when you are ready to use the system

again.  Alternatively, some programs temporarily lock the system

in a keyboard loop, waiting for a keypress to return to DOS.

ERROR CHECKING:

Many programs do not check for invalid track numbers, so if the

track assumptions are wrong, the programs don't know it and may

erroneously report successful parking.  Some of the programs

with error-checking confirm only that the selected track is

valid, while others confirm whether the actual parking is

successful.  "Success", however, doesn't necessarily mean to

the innermost cylinder -- it depends on whether the program's

track assumptions are correct.

Most of these programs move the drive heads via bios service

INT_13/0Ch ("Seek Cylinder"), though not all systems will flag

errors in this procedure (see Appendix I).  PARK-1, PARKHEAD,

and (in 8088 systems only) PCPARK10 instead use INT_13/04h

("Verify Sector"), which can reliably report errors, regardless

of system.  Ironically, PARK-1 doesn't check for errors, though.

Some programs will make several parking attempts, if necessary.

However, using a utility to trap hard disk interrupt calls

revealed none of the programs ever needed to make more than one

attempt on the test systems.  This could be different on an

old MFM or RLL disk that is starting to get out of alignment,

but in that case the disk may need a fresh low-level format

anyway, before more serious disk errors develop.

None of the programs are sensitive to the problem of wrapping

the parking track around the 1023 limit and back to a beginning

cylinder.

The programs that needed error-checking did not have it, and all

the programs with error-checking would have done just as well

without it, so whether a program has error-checking or not does

not seem to be a useful criterion to selecting a parking program.

MATH PROBLEM:

Programs that increment the number returned by the bios service

INT_13/08h routine must exercise care.  The track number is a

10-bit number encoded in an odd fashion, so a problem may occur

if incrementing crosses an exact multiple of 256.  PARK-1 and

PARKIT-2 did not seem to handle this properly.  Most programs

handle the problem in two or three simple lines of code, but

PARKIT-1 completely decodes the number of sectors and cylinders,

adds +1, then reencodes the numbers before parking - excessive

and inefficient, though it does circumvent the math bug.

SYSTEM-RELATED PROBLEMS:

PARKTIME and HDPARK-2 hung on the 8088 test systems, so should be

carefully tested if you have a PC or XT.  It's not clear whether

the problem is with the systems or the programs.  These are two

of the three programs in this roundup that display color.  The

8088 test systems used monochrome displays, so the problem may

be a conflict with 8088 monochrome systems in particular.  The

problem did not seem to occur on the 286, even with a monochrome

display.

The third color program, PARKIT-2, also did not work on the 8088

systems, but did not crash, either.  Other evidence suggests

PARKIT-2 may be reacting as intended to seek errors, not a video

conflict.

VIDPARK had problems with the 286/386/486 systems.  The TSR does

not take care to reenable the PIC (Interrupt Controller) hardware

when it activates, so the keyboard or TSR may lock up for awhile

until other bios activity eventually restores the PIC.  This did

not seem to affect the 8088 systems.

AUTOPARK and its companion, PARK-2, rely on track-seek errors to

locate the innermost cylinder, so are incompatible with systems

that do not return seek errors.

Similarly, PARKHEAD relies on finding an unformatted track.

While searching for an unformatted track on the 386's drive,

with 1024 cylinders, it looped back around to the beginning of

the disk in a never-ending quest to find a place to park.

For some reason, PCPARK10 hung on two of the 486 test systems.

Both had the CMOS Landing Zone (which PCPARK10 reads) set to 0,

but changing the CMOS had no effect - the program still hung.

Another problem is unique to the TSR parkers.  The bios INT_13

routine is called by programs when reading or writing to disk

drives, including the floppy drives, but it is not designed to

handle two INT_13 calls at the same time.  A single application

program by itself wouldn't do this, but if a TSR breaks in

while that application is accessing any drive, the conflict can

cause the system to crash.  An example of this would be a

parking TSR that times out while another program is reading a

floppy.  In this situation the TSR must not be allowed to

interrupt the other program's disk access already in progress.

Only AUTOPARK and DMGPARK prohibit this, each by putting its

counters on hold while another INT_13 is being processed.  This

prevents the TSR from timing out during another disk access,

albeit with a corresponding delay in the nominal timeout period.

VIDPARK and PDTIMPRK will blindly issue a conflicting INT_13

call, potentially crashing the system.  TIMEPARK, TPB, TESRPARK,

and PARKTIME also don't deal correctly with the concurrent

INT_13 problem.  These four parkers reset their counters at the

start of any INT_13 call, including (unwittingly, perhaps?)

floppy disk accesses.  This should avoid crashes because it's

not likely any single INT_13 call will still be in progress by

the time the counter, having been reset at the start of the

INT_13, eventually times out.  But note this approach also means

that frequently accessing any drive - even a floppy drive - will

prevent a hard disk from parking.

All TSRs except PARKTIME maintain some method of telling when

the disk has not been accessed since the parking operation, so

they do not continue "reparking".  PARKTIME instead continues to

reissue the parking command in intervals of the timeout period.

This is not significant, however - if the heads have not been

moved since the last parking operation, the repetitive commands

don't change anything.

As an aside, TPB's screen blanker did not blank the 286/386/486

test systems.  VIDPARK blanked the 286's monochrome screen, but

not the 386/486 VGA screens.

COMMENTS ON SPECIFIC PROGRAMS

-----------------------------

TIMEPARK: Timeout period 1-9 mins., unchangeable once installed.

Program assumes two disks, does not validate parking.

Maintains one countdown register, parking both disks

together.  Resets counter on any disk access, including

floppy drives, so repeated activity on any drive will

prevent hard disk(s) from parking.  Does not check for

prior installation, so multiple copies may be installed.

AUTOPARK: Parks one or two disks.  Timeout period 30 seconds,

cannot be set from command line.  Maintains one

countdown register, parking both disks together, so

continued activity on one disk will prevent the second

disk from parking.  Uses a trial-and-error method to

find the parking track, jumping back and forth across

the disk while it zeroes in on the last valid track -

inefficient and not very elegant, but is done only

when installed.  Hangs on any system that does not

return seek errors or has 1024 cylinders because it

can't find any "invalid" tracks.  Redundant code,

could be shorter with subroutines, though is still

one of the smaller programs here.

VIDPARK:  Parks one or two disks.  Does not validate parking.

Includes screen blanker.  Blanking and parking timeout

periods 1 min. to 1 hr., changeable after installation,

and can be disabled.  Help screen.  Maintains separate

countdown registers, parking each disk independently.

May break in while system is in the middle of another

disk access, crashing the system.  Does not handle the

Interrupt Controller hardware (PIC) properly, causing

keyboard and/or system lockup on 286, 386, and 486

systems.  Screen blanker didn't work on the 386/486s.

PDTIMPRK: Timeout period 1-9 mins., unchangeable once installed.

Program assumes 2 disks.  Maintains one countdown

register, parking both disks together, so continued

activity on one disk will prevent the second disk

from parking.  May break in while system is in the

middle of another disk access, crashing the system.

Does not check for prior installation, so multiple

copies may be installed.

TPB:      Program assumes 2 disks.  Includes help screen and

screen blanker.  Blanking and parking timeout periods

1-9 minutes, changeable after installation, blanking

can be disabled.  Maintains one countdown register,

parking both disks together.  Resets counter on any

disk access, including floppy drives, so repeated

activity on any drive will prevent hard disk(s) from

parking.  On test systems it parked the heads at the

second-innermost cylinder.  Blanker did not work on

the 286/386/486 test systems.

TESRPARK: Finds as many disks as are installed.  Help screen.

Does not validate parking.  Timeout period 1-9 min.,

changeable after installation.  Sends audible signal

when parker activates.  Maintains one countdown

register, parking all disks together.  Resets timer

on any disk access, including floppy drives, so

repeated activity on any drive will prevent hard

disk(s) from parking.  TSR includes hotkey that acts

as immediate parker.

PARKTIME: Parks one disk only.  Does not validate parking.  In

default mode it immediately parks disk and returns to

DOS.  Optional TSR mode includes on-screen clock and

allows timeout settings of 10 secs. or 1-9 minutes,

changeable after installation.  TSR conveniently does

not prevent running the program again as an immediate

parker.  Help screen.  Resets timer on any disk access,

including floppy drives, so repeated activity on any

drive will prevent hard disk from parking.  Clock is

nice, but can't be shut off.  Crashed on the 8088 test

systems.  Color.

DMGPARK:  Parks one or two disks.  Timeout period 1 second to

1 hour, changeable after installation, can be disabled.

Maintains separate countdown registers, parking each

disk independently.  Help screen.  The only program in

which the parking track can be overridden by specifying

it on the command line, during or after installation.

SHIPDISK: Tries to park 50 (!) cylinders further than the number

returned by INT_13/08h.  Does not validate parking.

Calls IBM ROM BASIC routine INT_82 (which few systems

have) for simple screen displays, so hangs on just

about everything!

DISKPARK: Finds as many disks as are installed.  Makes up to 3

attempts to park disks, validates parking.  Finishes

by mostly clearing screen and entering a keyboard

loop (press three CR's to return).  On test systems

it parked the heads at the second-innermost cylinder.

PARKAL:   Parks one or two disks.  Makes up to 3 attempts to park

disks, validates parking.  Finishes with a keyboard

loop (press any key to return).  On test systems it

parked the heads at the second-innermost cylinder.

PARK-1:   Parks one disk only.  Does not validate parking.

Potential math bug affects disks with a number of

cylinders near multiples of 256.  Finishes with

permanent lock requiring reboot if not powered down.

HDPARK-1: Parks 1 or 2 disks, validates parking.  Returns to DOS

after completion.  Due to a code error, if there are

3 or more disks, it will report "none" installed.

PARK-2:   Companion to AUTOPARK, using much of the same code.

Uses a trial-and-error method to find the parking

track, jumping back and forth across the disk while

it zeroes in on the last valid track -- inefficient,

and perhaps harder on the disk than any of the other

programs here.  Hangs on any system that does not

return seek errors or has 1024 cylinders because it

can't find any "invalid" tracks.  Redundant code,

could be shorter with subroutines, though is still

one of the smaller programs here.

PARK-3:   Parks 1 or 2 disks, validates parking.  Draws picture

of a power switch and returns to DOS after completion.

On test systems it parked the heads at the second-

innermost cylinder.

PCPARK10: Finds as many disks as are installed.  Makes up to 3

attempts to park disks, validates parking.  Fills

screen with text and draws picture of a power switch,

also supports 40-column CGA.  Finishes with keyboard

loop - default is permanent lock requiring reboot if

not powered down, but can be overridden with "/U"

command line parameter.  The only program to use CMOS

landing zone data on 286/386/486 systems.  Parks by

using INT_13/04h ("Validate Sector") in 8088 systems,

but uses INT_13/0Ch ("Seek Cylinder") when data is

taken from the CMOS.  For some reason, crashed on two

of the four IDE/486 systems.

PARK-4:   Parks 1 or 2 disks, does not validate parking.

Finishes with keyboard loop (press Esc to return).

PARK-5:   Program assumes two disks, does not validate parking.

Returns to DOS after completion.

PARKIT-1: Finishes with a permanent lock requiring reboot if not

powered down.  Inefficient, convoluted code - the size

and obscure code made it difficult to determine how

many disks it will park, or whether it checks for

parking errors.

PKANYD:   Program assumes two disks, does not validate parking.

Returns to DOS after completion.  On test systems it

parked the heads at the second-innermost cylinder.

NJPARK:   Finds as many disks as are installed.  Tries to park

8 cylinders further than the track indicated by

INT_13h/08h, no validation.  Finishes with a keyboard

loop (press any key to return).  Reported successful

parking on all test systems, but heads were not moved

on four systems and a fifth was parked at track 0.

HDPARK-2: Parks 1 to 4 disks.  Does not validate parking.  Ends

with a keyboard loop (press any key to return), while

colorful animated "bouncing floppy disks" move around

the screen.  Crashed on the 8088 test systems when

returning to DOS.  Apparently compiled with Turbo C.

Color.  Not very efficient code.

PARK-6:   Finds as many disks as are installed.  Does not validate

parking.  Finishes with a keyboard loop (Press any key

to return).

PARKIT-2: Finishes with a keyboard loop (press Esc to return).

Reports number of disks installed, but parks only the

first disk.  Aborted and returned to DOS on the 8088

and 486b test systems.  Since these were the only test

systems that should have returned a parking error, it

appears the program validates parking.  It parked the

386 (1024 cylinders) at track 768.  On each 486 test

system, screen displayed incorrect numbers for sectors

per track.  However, the problem seems to be limited

to the screen display - the parking calls contained

correct sector information.  Apparently compiled with

Turbo C.  Color.

PARK-7:   Parks 1 or 2 disks, does not validate parking.  Returns

to DOS after completion.  On test systems it parked the

heads at the second-innermost cylinder.  Probably the

sloppiest code in this survey, with several unused or

ill-used program segments.

PARKHEAD: Parks 1 or 2 disks.  Starts from same cylinder as the

"+1" parkers, but then finds last track by incrementing

or decrementing track number and looking for formatted,

verifiable sectors (via INT_13/04h).  Returns to DOS

after completion.  Hung on the 386 test system.

APPENDIX I -- THE TEST SYSTEMS

------------------------------

Each program was tested on eight systems, including an assortment

of MFM, RLL, and IDE drives and a variety of CPUs.  The test

systems are summarized below.  The four 486 systems all included

IDE drives with sector-translating controllers.

This data shows that a program cannot count on errors being

flagged when it calls for the parameters of a nonexistent disk

or attempting to move the heads to an invalid track.  Note most

of these systems reported parameters for a second disk, even

though there was no such disk installed.  When commanded to seek

to an invalid track (such as 1023 in most cases), some systems

simply moved to the last valid track, while the rest flagged an

error.

TABLE III - The Test Systems

test system     PC2    XT   286   386  486a  486b  486c  486d

CPU            8088  8088  i286  i386   iSX   iDX   iDX  iDX2

video          Herc   MDA  Herc   VGA   VGA   VGA   VGA   VGA

disk 1 size     20M   10M   20M  120M  200M  200M  120M  330M

disk 2 size      -     -    20M    -     -     -     -     -

disk 3 size      -     -     -     -     -     -     -     -

INT_13h/08h

reports tracks:

on 1st disk     613   304   613  1022   681   681   760  1008

on 2nd disk     ERR   304   613   304   613   304   304   304

on 3rd disk     ERR   ERR   ERR   ERR   ERR   ERR   ERR   ERR

INT_13h/1Ch

seeks to track:

on 1st disk     614   305  1023  1023  1023   682   764  1023

on 2nd disk     ERR   ERR  1023   ERR   ERR   ERR   ERR   ERR

Additional notes:

PC2:  PARKTIME and HDPARK-2 crashed.  NJPARK and PARKIT-2 did

not park system.  AUTOPARK and PARK-2 found track 614,

PARKHEAD found 615, but PARKIT-2's attempt at 615 aborted.

XT:   System did not report an error when asked for parameters

(INT_13h/08h) of a nonexistent second disk.  PARKTIME and

HDPARK-2 crashed.  NJPARK and PARKIT-2 did not park system.

AUTOPARK and PARK-2 found track 305, PARKHEAD found 306,

but PARKIT-2's attempt at 306 aborted.

286:  Two-disk system.  Either disk parked using any track number

(flagged no seek error, disks responded to any seek call).

AUTOPARK and PARK-2 hung, unable to find an invalid track

(controller returned no seek errors).  VIDPARK didn't hang,

but never activated, either.  CMOS Landing Zone set to 615

for both disks.  PARKHEAD parked at 614.

386:  System did not report an error when asked for parameters

(INT_13h/08h) of a nonexistent second disk.  Could not

test whether system would flag seek error to nonexistent

track (no invalid track number on this disk).  PARKHEAD

hung, unable to find an unformatted track.  AUTOPARK and

PARK-2 hung, unable to find an invalid track.  NJPARK

parked system at track 0.  PARKIT-2 parked at track 768.

VIDPARK hung.  CMOS Landing Zone set to 1023.

486a: System did not report an error when asked for parameters

(INT_13h/08h) of a nonexistent second disk.  Parked using

any track number (flagged no seek error, disk responded to

any seek call).  AUTOPARK and PARK-2 hung, unable to find

an invalid track.  VIDPARK hung.  CMOS Landing Zone set to

683.  PCPARK10, PARKIT-2, and PARKHEAD parked at track 683.

486b: System did not report an error when asked for parameters

(INT_13h/08h) of a nonexistent second disk.  VIDPARK and

PCPARK10 hung.  CMOS Landing Zone set to 0.  AUTOPARK,

PARK-2, and PARKHEAD parked at track 682.

486c: System did not report an error when asked for parameters

(INT_13h/08h) of a nonexistent second disk.  VIDPARK hung.

CMOS Landing Zone set to 762.  PCPARK10 and PARKIT-2 parked

at track 762.  AUTOPARK, PARK-2, and PARKHEAD found valid

track at 764 and parked there.

486d: System did not report an error when asked for parameters

(INT_13h/08h) of a nonexistent second disk.  Parked using

any track number (flagged no seek error, disk responded to

any seek call).  AUTOPARK and PARK-2 hung, unable to find

an invalid track.  VIDPARK and PCPARK10 hung.  PARKHEAD

parked at track 1009.  CMOS Landing Zone set to 0.  This

system's drive (Western Digital Caviar 2340) automatically

shut off its drive motor when the drive heads were moved

to track 1010 or beyond.  Only DMGPARK and PARKIT-2 were

able to accomplish this.

APPENDIX II -- The SEEK Utilities

---------------------------------

This report is being distributed with three utility files to

help you assess how your system responds to the bios service

routines the parking programs use.  The three programs are

identical except for the drive that each interacts with.

SEEK80.COM works with the first hard disk in your system,

SEEK81.COM works with the second, and SEEK82.COM works with

the third.  The SEEK programs consist of two parts.

The first part shows the number of cylinders on the hard disk

that your system will report in response to a call to Bios

Interrupt service 13h, function 08h ("Get Parameters").  This

number can be used to evaluate which cylinder these parking

programs will try to use as the parking track.  For example,

if SEEK80 indicates 613, the programs listed as "+0" in Table

II would try to use track 613 as the parking track, while the

programs listed as "+1" would try to use track 614.

If the SEEK program is run with a number on the command line,

the second part of the utility will attempt to move the drive's

heads to that cylinder.  For example, entering "SEEK80 614"

will try to move the heads on the first hard disk to track 614.

The utility will report if an error code is reported by the

system.  Thus, you can evaluate whether the parking track a

program wants to use is a valid track.

By trying different numbers, you should be able to get an idea

of how your system's responses compare to the test systems used

for this report.  Also, you may find it instructive to run the

SEEK programs from a floppy or ramdisk (so the heads will not

get unparked when returning to DOS) and watch the responses of

your hard drive access light.  For example, entering the command

"SEEK80 614" after the heads have already been moved to track 613

would result in a brief flash from the drive access LED, while

"SEEK80 0" should result in a longer flash of the drive LED and

a more noticeable sound from head movement.  This technique was

used to help evaluate whether programs such as NJPARK actually

moved the heads or not.

Note: By using a parking track number on the command line the

SEEK programs essentially become immediate-action parkers.  In

comparison to the programs in Table II, the SEEK programs would

park only the one drive involved and would not lock the system

in any keyboard loop.

DISCLAIMER:

No warranties, expressed or implied, are made with the SEEK

programs, and the author assumes no liability for their use.

While this report is copyrighted, the SEEK programs are hereby

released into the public domain, and may be freely copied,

distributed, or modified.

This review has tested the programs on a sampling of MFM, RLL,

and IDE drives.  I have not tested them on ESDI or SCSI drives.

Various memory managers were represented among the test systems,

including EMM386, QEMM, and 386MAX.  SMARTDRIVE provided disk

caching on some systems.  None of the test systems used special

drivers like Disk Manager, such as might be used with drives

exceeding 1024 cylinders.

I have tried to take care in confirming the data in this report,

including repeating all tests, writing an interrupt-trapping TSR

to step through each program, and disassembling portions of the

code in most programs to see if the code confirms my findings.

However, I did not confirm reasons for all problems discovered.

Copyright 1992, 1994, All rights reserved

by Dan Goodell

1261 Hookston Road

Concord, CA  94518

CompuServe: 71520,3116

This text is the copyrighted work of the author, Dan Goodell.

It may not be republished, in whole or in part, without the

expressed, written permission of the author, except that it

may be freely distributed without charge for private, non-

commercial use, subject to the following:

*  The text may not be modified.

*  It must be distributed with the accompanying SEEK programs

in unmodified form.  Although the SEEK programs are public

domain, they must be unmodified for distribution with this

file.

*  No fee may be made for distribution, except for a nominal

disk copying charge.